\# Scan2Plan Standards Reference v1.0

\---

\#\#\# \#ScanningStandards

\#\#\#\# Levels of Accuracy (LoA)

\- \*\*LoA 40 (Measured Accuracy):\*\*

\- \*\*Tolerance:\*\* 0–1/8 inch.

\- \*\*Application:\*\* Point clouds generated directly from laser scans.

\- \*\*Purpose:\*\* Provides high-precision scan data aligned with real-world conditions.

\- \*\*Validation Methods:\*\*

\- \*\*B-Validation:\*\* Overlapping scan alignment for consistency.

\- \*\*C-Validation:\*\* Control points aligned with survey data.

\- \*\*LoA 30 (Modeled Accuracy):\*\*

\- \*\*Tolerance:\*\* 0–1/2 inch.

\- \*\*Application:\*\* BIM/CAD models derived from point clouds.

\- \*\*Purpose:\*\* Ensures modeled geometry reflects scanned data for construction-ready deliverables.

\#\#\#\# Scanning Devices

\- \*\*Trimble X7:\*\*

\- \*\*Accuracy:\*\* ±2mm.

\- \*\*Features:\*\* Colorized scans, real-time registration.

\- \*\*NavVis VLX 2:\*\*

\- \*\*Accuracy:\*\* Up to 6mm.

\- \*\*Features:\*\* Mobile SLAM scanning for dynamic environments.

\#\#\#\# Scanning Process

\- \*\*Pre-Scan:\*\*

\- Conduct site surveys to identify access points and conditions.

\- Ensure comprehensive planning to cover all required areas.

\- \*\*On-Site Execution:\*\*

\- Perform live review to verify completeness of scan coverage.

\- Address missing data before leaving the site.

\---

\#\#\# \#ModelingStandards

\#\#\#\# Levels of Detail (LoD)

\- \*\*LoD 200:\*\*

\- \*\*Use Case:\*\* Schematic massing, basic geometry for feasibility studies.

\- \*\*LoD 300:\*\*

\- \*\*Use Case:\*\* Construction-ready elements, accurate geometry, and dimensions.

\- \*\*LoD 350:\*\*

\- \*\*Use Case:\*\* Detailed construction-ready models with joint and connection details.

\- \*\*LoD 350+ (HBIM):\*\*

\- \*\*Use Case:\*\* Custom detail for historic preservation, including carvings and ornate features.

\#\#\#\# Disciplines and Key Modeling Elements

\- \*\*Architecture:\*\*

\- \*\*Elements:\*\* Walls, doors, windows, ceilings, and floors.

\- \*\*Special Features:\*\* Custom details for historic and high-profile projects (LoD 350+).

\- \*\*Structure:\*\*

\- \*\*Elements:\*\* Beams, columns, trusses, and foundations.

\- \*\*Special Features:\*\* Detailed connections supporting structural analysis.

\- \*\*MEPF (Mechanical, Electrical, Plumbing, Fire):\*\*

\- \*\*Elements:\*\* Ducts, pipes, conduits, and sprinkler systems.

\- \*\*Special Features:\*\* Fully clash-detectable layouts compatible with multidisciplinary coordination.

\- \*\*Landscape:\*\*

\- \*\*Elements:\*\* Topography, pathways, and foliage.

\- \*\*Special Features:\*\* Georeferenced models for landscape design and historic preservation at LoD 350+.

\#\#\#\# Validation and Quality Control

\- \*\*3-Stage QC Process:\*\*

1\. Dedicated BIM Modeling and CAD drafting teams.

2\. Dedicated BIM and CAD QC teams.

3\. Senior Engineer Review.